# Project Deliverable Report Instructions

This document is a template for the project deliverable submissions. Your group will edit this document all semester and submit it each time you have an updated section (new deliverable is done). The goal is to keep everything in one place to be able to refer back to previous work and save you some time formatting a new document for every deliverable. Please keep track changes ON so that the TA can see what has been changed every time it gets submitted. So that it does not become laggy when the document is too large, 2 templates are provided (PD B-D and E-I).

Template conventions:

- Remove all red text, it is only there to guide you

- Remove this page (instructions)

- Replace all instances of <xxx> with the appropriate information for your group, for example you could replace <GROUP NUMBER> by ‘B1.3’

GNG2101

Design Project Progress Update

Waterproof Collar Team, Group B2.3

Submitted by:

Aisven Devanand, 300143937

Gavin Garfinkel, 300354474

Matthew George, 300356554

Rayhan Karim, 300377828

Jonah Philip, 300373587

November 19th, 2024

University of Ottawa

Table of Contents

Project Deliverable Report Instructions	i

Table of Contents	ii

List of Figures	iv

List of Tables	v

List of Acronyms and Glossary	vi

1	Introduction	1

2	Prototype 1, Project Progress Presentation, Peer Feedback and Team Dynamics	2

2.1	Prototype 1	2

2.1.1	Critical Assumptions to Test	2

2.1.2	Creating the Prototype	3

2.1.3	Prototype Testing	6

2.2	Project Progress Presentation	7

2.3	Project Plan Update – PD E	7

3	Design Constraints and Prototype 2	8

3.1	Design constraints	8

3.1.1	Non-Functional Design Constraints	8

3.1.2	Updated Detailed Design	8

3.2	Prototype 2 & 2.5	9

3.2.1	Summary of Prototype 1 Results	9

3.2.2	Critical Assumptions to Test in Prototype 2	10

3.2.3	Creating and Testing Prototype 2	10

3.2.4	Client Meet 3 Feedback	13

3.2.5	Creating and Testing Prototype 2.5	14

3.2.6	Next Steps for Final Prototype	16

3.3	Project Plan Update – PD F	17

4	Economic and IP Considerations	18

4.1	Economics report	18

4.1.1	List of Expenses	18

4.1.2	Income Statement	18

4.1.3	Break-Even Analysis	19

4.1.4	Assumptions for Economics Analysis	19

4.2	Intellectual property report	20

4.3	Project Plan Update – PD G	21

5	Design Day Pitch and Final Prototype Evaluation	23

6	Video and User Manual	24

6.1	Video pitch	24

6.2	User manual	24

7	Conclusions	25

8	Bibliography	25

List of Figures

Figure 1. Prototype 1 front, back, and side views	4

Figure 2. Prototype 1 rolled into a smaller package for portability	4

Figure 3. Different collar sizes	5

Figure 4. Front, back, and top view of the prototype with the vinyl layer	5

Figure 5. Updated design of soft cervical collar	9

Figure 6. Prototype 2	11

Figure 7. Prototype 2 Waterproof Test Before and After (middle and right)	12

Figure 8. Prototype 2.5	14

Figure 9. Prototype 2.5 Waterproof Test Before and After (middle and right)	15

Figure 10. Cervical collar patent by George W. Cottrell [3]	20

Figure 11. Sports neck protector patent by Stephen Marston and Isaac Aboud [4]	21

List of Tables

Table 1. Acronyms	vi

Table 2. Glossary	vi

Table 3. Summary of prototype testing	6

List of Acronyms and Glossary

Provide a list of acronyms and associated literal translations used within the document. List the acronyms in alphabetical order using a tabular format as depicted below.

Table 1. Acronyms

| Acronym | Definition |
| --- | --- |

Provide clear and concise definitions for terms used in this document that may be unfamiliar to readers of the document. Terms are to be listed in alphabetical order.

Table 2. Glossary

| Term | Acronym | Definition |
| --- | --- | --- |

# Introduction

Our team’s goal is to design a soft cervical collar that is water resistant and can handle many different activities such as Aikido and swimming. There is also the goal of the collar being able to withstand harsh chemical conditions such as chlorinated water and washing machines to be rewashable and reusable. We have developed two concepts, one ‘traditional’ and one ‘alien’ with unique features, and have made one final concept that incorporates both designs that feature holes in the cervical collar for extra breathability and comfort. This document will go over prototype development, the progress of the project, design constraints, an updated second prototype, economic and IP considerations that contain an economics report and intellectual property report, the design day pitch and final evaluation of the developed prototype, and finally a video recording of the pitch with a user manual.

# Prototype 1, Project Progress Presentation, Peer Feedback and Team Dynamics

## Prototype 1

### Critical Assumptions to Test

When designing prototype 1, we determined three important critical assumptions that we could test for: the qualitative rigidity of the collar, the quality of the Velcro strips, and the ease of rollability and compression.

Our most critical assumption is that the polyurethane foam would be rigid enough to alleviate the stress from the neck by supporting the chin, but still be flexible enough for a reasonable range of motion. To test this, users will wear the collar and describe their experience, focusing on two factors: whether there is a reduction in neck strain, and if they can move their head a reasonable amount on all three axes. Since our client cannot participate in these tests, the users will serve as proxies. Their feedback will provide qualitative insight into how effective the collar is at supporting the head and reducing neck pressure. This test ensures the final prototype will be designed for comfort, which cannot be quantitatively measured, and designed for quality, if the collar does not alleviate enough strain on the neck, then it is not useful.

Another critical assumption is that the collar could be easily stored by rolling it up and compressing it to reduce its volume. To test this, the prototype will be rolled up and compressed 5 times, and the volume reduction will be measured to gauge portability. Following this, the material will be inspected for any signs of wear, such as creasing or loss of rigidity. This ensures that the collar is designed for portability, as testing the ease of rolling and compressing the collar directly ties to its portability, a key requirement for users who need to store or transport it frequently. It also ensures the collar is designed for accessibility, as rolling and compressing the collar for storage needs to be convenient for a user who has physical limitations.

A final critical assumption this prototype is designed to test for is the viability of Velcro for use in the collar. To test this, we devised two tests:

- Peeling Strength: The Velcro will be peeled and reattached multiple times to assess how well it holds over repeated use.

- Underwater Test: The collar will be submerged in water for 10 minutes to test how the Velcro holds in aquatic environments, simulating conditions the client will encounter during swimming.

These two tests ensure the collar is designed for accessibility as the ease with which the Velcro can be peeled and reattached affects how accessible and user-friendly the collar is, particularly for individuals with physical limitations. It also ensures that the collar is designed for quality, as testing the Velcro’s strength and durability ensures the collar meets high standards.

### Creating the Prototype

Using the three critical assumptions outlined in 2.1.1, we designed a prototype to test them. The prototype was made using items found in our houses such as sponges, epoxy glue, and Velcro straps. The sponge was cut and glued using epoxy glue following the general shape of the detailed design. Three holes were added to the front to provide breathability and to test the compressibility. The hook portion of the Velcro strap was cut in half and glued to the two ends of the collar, with the hook side facing outward. The loop portion of the Velcro was then placed on the hooks, providing adjustability to the collar. The prototype can be seen in Figure 1. A smaller hook portion of the Velcro was added to the front for the portability feature, as seen in Figure 2. In the rolled-up configuration, the collar is 6 inches in diameter and has a height of 4 inches. The prototype can also be worn by individuals with neck diameters ranging from 4 to 5 inches. Figure 3 shows the different sizes that the collar can accommodate. This, however, reduces the strength of the Velcro as there will be less contact between the hooks and loops.

Figure 1. Prototype 1 front, back, and side views

Figure 2. Prototype 1 rolled into a smaller package for portability

Figure 3. Different collar sizes

The prototype was also covered in Vinyl sheets and held together with tape to simulate the silicone layer from the detailed design. During the tests, it was noticed that the vinyl layer added more rigidity to the collar, was uncomfortable to wear, and did not provide any waterproofing. The vinyl layer was subsequently removed from the prototype to allow for the collar to dry and it was later decided by the team to scrap this component from the prototype. As such, this prototype will only include the foam and Velcro components of the design for preliminary testing and future prototype(s) will be made to test the silicone component. Figure 4 shows the cervical collar prototype with the vinyl layer.

Figure 4. Front, back, and top view of the prototype with the vinyl layer

### Prototype Testing

Table 3. Summary of prototype testing

| Test Number | Test Name | Method of Data Collection | Test Plan | Test Results |
| --- | --- | --- | --- | --- |
| 1.1 | Rigidity | Qualitative | The brace is compressed, folded, and when no holes were created (for breathability.) Three holes are cut out; one in the middle and two on either side. Then, the brace will undergo the same physical test conditions. Vinyl sheets will then be wrapped around the brace and the same test conditions will be applied. | - Compressibility is more noticeable when holes are added. - The vinyl sheets reduce the flexibility. |
| 1.2 | Velcro Strips | Qualitative/ Quantitative | Repeated removal and reconnection of the Velcro to see if it will deform under water. The collar will also be submerged in water for 10 minutes. | - The Velcro kept the same strength and showed no signs of wear or tear throughout the repeated removal of the strips. - The water made no difference to the strips after the time lapsed. |
| 1.3 | Ease of rollability and compressibility | Quantitative | The collar will be rolled up and compressed five times. The dimensions will be recorded | - When rolled up the collar is 6 inches in diameter and 4 inches in height. |
| 1.4 | Performance in Water | Qualitative | The brace is covered with vinyl sheets. The brace is submerged in water for 45 minutes. The brace is then tested for water retention, compressibility differences, and weight differences. | - The vinyl sheet was not fully waterproof and allowed water to seep through - All other factors were not tested since the brace absorbed more water than anticipated (the vinyl sheet failed) |

## Project Progress Presentation

https://uottawa-my.sharepoint.com/:p:/r/personal/mgeor060_uottawa_ca/_layouts/15/doc.aspx?sourcedoc=%7Bb89e8db5-ff31-4e96-8143-2a4c9a049f12%7D&action=edit

## Project Plan Update – PD E

# Design Constraints and Prototype 2

## Design constraints

### Non-Functional Design Constraints

- Durability: Frequent bending, stretching, and ordinary wear and tear should not cause it to lose its structure or usefulness.

- Maintenance and washability: The collar must be machine washable and durable enough to last several washings without losing its quality. To make maintenance easier, the materials chosen should be resistant to mold growth and water absorption.

- Weight and Portability: To make sure the user is not burdened when wearing the collar in the water, it should be lightweight (targeting 250–500 grams). Its portability should be increased by making it simple to fold or roll up for storage and travel.

- Comfort and Flexibility: A wearable product's collar should be soft, breathable, and flexible enough to adjust to the user's movements because comfort is crucial.

- Heat Resistance: To guarantee durability in hot tub settings, the collar should be able to tolerate temperatures of up to 60°C for up to an hour.

- Adjustability: The collar should be able to adjust depending on if the client wants a loose or tight fit.

### Updated Detailed Design

The detailed design was updated to include one Velcro strap at the back and another smaller piece used to lock the collar in its compressed form for portability, which was seen in prototype 1. The edges were also adjusted to have a smaller radius since the silicone layer of the collar will no longer be casted in a mold as will be shown in prototype 2 where the foam absorbed the silicone that was poured in the mold. Since the silicone will be coated manually by hand, it will be difficult to achieve the smooth edges shown in the initial design. Lastly, the color was updated to match with prototype 2.5’s exterior.

Figure 5. Updated design of soft cervical collar

## Prototype 2 & 2.5

### Summary of Prototype 1 Results

Rigidity: Adding holes improved breathability but led to noticeable compressibility, 		reducing flexibility. The vinyl sheets added rigidity but also restricted flexibility.

Velcro Strips: Even after being repeatedly taken off and put back together and immersed in 	 water for ten minutes, the Velcro showed no evidence of deterioration or wear. Velcro 		performance was not impacted by exposure to water.

Ease of Rollability and Compressibility: The collar's dimensions after rolling up were 6 		inches in diameter and 4 inches in height, demonstrating its portability and compactness.

Performance in Water: Water seepage resulted from the vinyl sheets' incomplete resistance 	 against water penetration. Since the vinyl absorbed more water than expected, impairing 		waterproof performance, this failure prevented additional testing of compressibility and 		weight differences following submersion.

### Critical Assumptions to Test in Prototype 2

- How does the piece of silicone feel on the skin

- How effective is the Velcro adhesive on the silicone surface

- Does the silicone layer make the foam waterproof

- How flexible/compressible is the silicone covered foam

- How does flexibility/compressibility compare to prototype 1 and client’s other collars?

### Creating and Testing Prototype 2

Using polyurethane foam and cutting it into a small piece, around half a foot x foot in area, the silicone making kit was used to create the silicone solution to pour. The polyurethane foam was placed into a molding box made from cardboard, fixed together with tape at the edges to prevent leakage. The silicone solution was then poured into the mold with hopes of covering the entire piece, however two critical problems arose. The first problem was since the foam was very porous, it soaked up most of the silicone poured into the box, the second problem was that the cardboard was not rigid enough, so it bulged out on one side, causing over thickness of the silicone layer. After the silicone was finished curing, the cardboard mould was removed and there was a nice smooth silicone layer around the base of the foam, with a very thin layer on the bottom. The biggest issue with this second prototype was that the silicone was absorbed into the foam and hardened the foam, making it very rigid. To fix these manufacturing flaws, in our next prototype we will first coat the foam with silicone and let it cure, to prevent silicone from leaking into the foam when the actual layer is poured around it. Additionally, we learned that we do not need much silicone, so we reduced our thickness criteria by a factor of 5 from 5 mm to 1 mm. Afterwards the excess foam was cut to test the prototype

Figure 6. Prototype 2

Figure 7. Prototype 2 Waterproof Test Before and After (middle and right)

| Test Number | Test Name | Method of Data Collection | Test Plan | Test Results |
| --- | --- | --- | --- | --- |
| 2.1 | Rigidity/Compressibility | Qualitative/Quantitative | See visually the compressibility of the silicone/foam block and measure compressibility with weights on top | Very rigid structure, compressed 0mm with a 1 kg weight on top, however this also indicates high durability, but is troublesome for activities such as aikido and swimming |
| 2.2 | Performance in Water | Qualitative/Quantitative | The bottom half containing the fully sealed silicone layer is submerged in water for 45 minutes, afterwards test difference in weight and qualitatively observe if there are any structural deformities | Huge success, zero structural deformities and exact same weight after being submerged in water |
| 2.3 | Velcro Adhesive Performance | Qualitative/Quantitative | Place the adhesive side of a piece of Velcro (hooks) on the Silicone surface of the prototype, and pull on it using a Velcro strip (loops) with a force of at most 4.2lbs [1] | This test failed since the Velcro adhesive did not stick on the Silicone surface and easily came off when it was lightly pulled |

### Client Meet 3 Feedback

Material and Comfort Needs: The client finds silicone's absorption qualities fascinating, but they are worried about it sticking to their skin or clothes. Instead of being overly soft like the sponges used today, they want a material that offers support without sacrificing comfort or movement. For ease of use in water, silicone should have a low buoyancy. The client wants the collar to be lightweight as the prototype was too heavy when he felt it.

Range of Motion and Support Balance: Since the client's range of motion varies every day and it is easier to turn their head to the left than the right, it is crucial that the collar provides flexibility of movement. In contrast to plastic waterproof collars that feel overly hard, the collar should not hinder head mobility. The customer wants a well-balanced design that provides support without sacrificing flexibility.

Design Integration and Durability: The client wants a collar that goes well with sportswear so that it feels and looks natural when exercising. They stress the need of using quality Velcro that is long-lasting, doesn't deteriorate with time, and offers dependable fastening and simple adjustment.

### Creating and Testing Prototype 2.5

After the failed attempt to construct prototype 2 since the Silicone was absorbed by the foam, the manufacturing process was updated. The following steps were taken to produce prototype 2.5:

- A small piece of foam (4 cm x 4 cm x 5 cm) was cut from the large sheet

- The Silicone mixture was made (by mixing liquids A and B) and a thin layer (1 mm) was poured over a flat surface

- The foam was placed on the layer of Silicone on one of its sides and let to cure for 2-3 hours

- The previous step was repeated for the 5 other surfaces

- Silicone was coated with a spatula on any surfaces that weren’t properly coated and left to cure

Figure 8. Prototype 2.5

The same tests were performed on Prototype 2.5 to see if the new manufacturing process would fix the issues seen in Prototype 2 while also maintaining the positive results of Prototype 2.

Figure 9. Prototype 2.5 Waterproof Test Before and After (middle and right)

| Test Number | Test Name | Method of Data Collection | Test Plan | Test Results |
| --- | --- | --- | --- | --- |
| 3.1 | Rigidity/Compressibility | Qualitative/Quantitative | See visually the compressibility of the silicone/foam block and measure compressibility with weights on top | Prototype 2.5 is compressible and holds its shape but is stiffer than Prototype 1. It compressed 5mm with a 10lb weight on top (Typical weight of human head [2]) |
| 3.2 | Performance in Water | Qualitative/Quantitative | The entire silicone covered foam is submerged in water for 45 minutes (while keeping it held down since it floats), afterwards test difference in weight and qualitatively observe if there are any structural deformities | Huge success, zero structural deformities and exact same weight after being submerged in water |
| 3.3 | Velcro Adhesive Performance | Qualitative/Quantitative | Place the adhesive side of a piece of Velcro (hooks) on the Silicone surface of the prototype, and pull on it using a Velcro strip (loops) with a force of at most 4.2lbs [1] | This test failed since the Velcro adhesive did not stick on the Silicone surface and easily came off when it was lightly pulled |

### Next Steps for Final Prototype

So far there has been much success in the prototypes and much has been learned about materials, properties and manufacturing. One of the main issues that needs to be resolved is securing the Velcro on the collar. Testing Prototypes 2 and 2.5 has shown that the adhesive on the Velcro does not stick to the Silicone surface. One possible solution that is being reviewed is to sew the Velcro on the foam body and cover the suture holes with Silicone. This will be tested on Prototype 2.5 before implementing it into the final prototype. Additionally, following the latest client meet, it was noted by the client that the Silicone would have high friction on the user’s skin which may be uncomfortable when swimming. The current solution for this problem is to add a wetsuit fabric over the silicone to improve skin comfort.

Following the prototype tests and client feedback, our next step will be to create a high-fidelity prototype, which focuses on the manufacturing process for the final design. This prototype will highlight the lessons learned from the previous prototypes and aim to create a fully functional design while also fixing the remaining issues. Our goal for this prototype is to create a collar which could pass off as the final design but will be able to teach us if there are any flaws in the design. This means that quality materials will be used, and much emphasis will be applied to its creation. By the end of this prototype, our group will have all the necessary information to create a design which meets all target requirements.

## Project Plan Update – PD F

# Economic and IP Considerations

## Economics report

### List of Expenses

- Materials (direct, fixed cost)

  - $10 of foam per cervical collar

  - $20 of silicone per cervical collar

  - $5 of fabric per cervical collar

  - Total Material cost of $35 per cervical collar

  - Expected to make 5*200 = 1000 collars/day = $700,000 materials cost/month

- Labour (direct, variable cost)

  - Employ around 200 people, pay minimum wage of $15/hour (variable as number of employees and wage can change), it is expected that 5 collars will be produced for each person with the help of machinery in one day, given 8-hour shifts, the cost will be $480,000 / month

- Overhead (indirect, variable cost)

  - Approximately $720,000/month in overhead costs

- Advertising (indirect, fixed cost)

  - $21,600/month will be allocated to advertising the cervical collar

### Income Statement

The cervical collar will be sold at $100/collar

Income Statement for a Period of 3 Years

Sales Revenue: $72,000,000

Costs of Good Sold: $(25,200,000)

Operating Expenses: $(43,977,600)

Operating Net Income: $2,882,400

### Break-Even Analysis

To perform the break-even analysis, the net positive values of the expenses and incomes for the next 3 years need to be determined. It is assumed that the company needs to make the same number of collars each year (N). As such the break-even point will be three times the annual number of collars needed to be produced. Similarly, all expenses are assumed to be the same each year. An annual interest rate of 5.95% will be used since it is the good approximation of the current annual interest rate in Canada [3]. The cash flow diagram for the expenses can be seen in Figure 10 while the cash flow diagram for the incomes can be seen in Figure 11. For the expenses diagram, it was assumed that all expenses were made at the start of the year, while for the incomes diagram, the collars that were made were sold at the end of the year.

Figure 10. Expenses cash flow diagram

Figure 11. Incomes cash flow diagram

To calculate the expenses and incomes, an NPV analysis was done, as seen below:

EX1=35$collar+8hrday∙15$hr5collarday∙N+720,000$month+21600$month∙12=59∙N+8,899,200

Expenses=EX1+EX2+EX3=EX1+EX11.0595+EX11.05952=59∙N+8,899,200+59∙N+8,899,2001.0595+59∙N+8,899,2001.12254025=167.25∙N+25,226,367.24

Incomes=IC1+IC2+IC3=100$collar1.0595+100$collar1.05952+100$collar1.05953∙N=267.55∙N

Using the break-even point equation, we can calculate the number of collars that needs to be made in one year:

Cbuy∙N=Cmake∙N+Fmake↔267.55∙N=167.25∙N+25,226,367.24
∴Cbuy=$267.55, Cmake=$167.25, Fmake=$25,226,367.24

N=FmakeCbuy-Cmake=25,226,367.24267.55-167.25=251510 collarsyear

To make a profit, the company needs to sell at least 251510 collars every year.

### Assumptions for Economics Analysis

- The overhead cost will be around 36% of the sales [4]. We expect each employee to produce 5 cervical collars per day with the help of factory equipment, putting our sales at $2,000,000 per month if we sell all of the cervical collars. 36% of $1,600,000 = $720,000.

- We expect to have over $700,000 monthly in sales, we should spend 2-5% on marketing, so advertising will be around 3% = $21,600 a month [5].

## Intellectual property report

The first patented neck collar was the cervical collar by George W. Cottrell, patented on November 15, 1966. The invention uses an inflatable balloon within the collar to apply pneumatic pressure and provide comfort and support to the wearer [6]. This patented design is relevant to our problem description since the inflatable nature of this collar essentially makes this a waterproof neck collar since it does not rely on the compressibility of foams to provide comfort.

Figure 12. Cervical collar patent by George W. Cottrell [6]

Another patented cervical collar design was the sports neck protector by Stephen Marston and Isaac Aboud which received its patent on August 18, 1987 [7]. This patent was selected since it uses a design provides comfort while doing sports activities. The client specified he wanted his collar to be used in swimming and aikido.

Figure 13. Sports neck protector patent by Stephen Marston and Isaac Aboud [7]

## Project Plan Update – PD G

# Design Day Pitch and Final Prototype Evaluation

## Problem statement, whom we are solving it for and why we are solving it

Imagine waking up every day with a condition that affects your balance and prevents you from enjoying activities that most of us take for granted. Vestibular damage impacts thousands of people, causing severe balance issues and pain that makes daily tasks—especially physical activity—challenging and unsafe. Our client doesn’t have to imagine that; he lives through them every day. As someone who loves to swim, do Aikido, and soak in hot tubs, our client was born to be active; however, he often faces unique challenges due to his condition, requiring a cervical collar that supports his neck without discomfort or joint inflammation. Current collars on the market are often too firm, bulky, or unsuitable for moisture-prone environments like swimming pools or hot tubs. The options out there just didn’t fit his active lifestyle, leaving him with constant compromises. Our client needed a collar that provides support, is breathable, easy to clean, and adaptable for use in water or on land.

## Our solution fleshed out

Our solution is a soft cervical collar that meets these needs while offering unique features: it's waterproof, chlorine resistant, and machine washable. Its design ensures balance between support and flexibility, helping prevent feelings of vertigo and pain. Unlike traditional collars, ours is designed for all-day wear and active use. It’s also lightweight, adjustable in tightness, rollable for easy storage, and extremely comfortable.

## Physical demo

- Here, you can see how the collar maintains rigidity while bending for comfort. (Bend it)

- Notice its resistance to water and how easy it is to clean. (Dunk in water)

- We’ve also ensured it’s lightweight and breathable for maximum user comfort. (Show holes)

- We’ve also ensured it’s fully rollable for convenient storage and transport, addressing another common user complaint. (Roll and unroll it)

## Challenges overcome

One of the biggest hurdles was creating a material that could perform reliably in both chlorinated water and dry conditions while remaining comfortable. Our solution was to take a semi-rigid piece of foam and coat it in a thin layer of silicone. This ensures the collar is airtight whilst remaining lightweight and comfortable on the skin. We tested it in various conditions, and the results are very promising.

## Call to action

Our prototype empowers users like our client to regain independence and safety in their daily lives, whether they’re at home, in a pool, or enjoying recreational activities. With your support, we can make this solution available to others who need it.

# Video and User Manual

## Video pitch

Add link to video.

## User manual

See separate template for the user manual. Do not write the content here.

# Conclusions

The cervical collar prototype one was created to test important hypotheses about Velcro strap performance, portability, and rigidity. Its performance was assessed by a number of tests, such as how well the collar relieved neck tension, how compressible it was for storage, and how strong the Velcro was in damp environments. The outcomes showed both places for improvement and areas of achievement. The polyurethane foam used for the collar shown decreased flexibility and compressibility, particularly when holes were introduced for breathability, whereas the Velcro retained its strength and durability even after frequent usage and exposure to water. Furthermore, the different neck widths could be handled by the collar's design, doing so made the Velcro less secure. A serious defect was discovered during the waterproof test which was that the vinyl covering was not completely waterproof, allowing water to flow through. In conclusion, the prototype showed a strong basis for addressing accessibility, comfort, and portability; nevertheless, more iterations are needed to improve material selections and guarantee overall longevity, particularly in damp conditions.

# Bibliography

[1]	“How do VELCRO® Brand Fasteners Work?,” USA, 23-Feb-2023. [Online]. Available: https://www.velcro.com/news-and-blog/2023/02/how-do-velcro-brand-fasteners-work/.

[2]	“How much does your head weigh?,” GW Osteopathy, 28-Jan-2018. [Online]. Available: https://www.gwosteopathy.co.uk/much-head-weigh/.

[3]	J. David, “Prime rate in Canada,” Ratehub.ca, 06-Jul-2022. [Online]. Available: https://www.ratehub.ca/prime-rate.

[4]	Markitors, “Overhead costs for small businesses: Calculate & reduce,” LendThrive, 30-May-2022. [Online]. Available: https://lendthrive.com/blog/overhead-costs-for-small-businesses.

[5]	“What is the average marketing budget for a small business?,” BDC.ca, 12-Sep-2020. [Online]. Available: https://www.bdc.ca/en/articles-tools/marketing-sales-export/marketing/what-average-marketing-budget-for-small-business.

[6]	G. W. Cottrell, “Cervical collar,” 3285244, 15-Nov-1966.

[7]	S. Marston and I. Aboud, “Sports neck protector,” 4686710, 18-Aug-1987.