Conceptual Design

Brooke Joyce, Hassan Ahmed, Ancelin Avenido, Jacky Zhu, Christopher Webster

February 13, 2022

Table of Contents

*DISCLAIMER*: We included functional, non-functional requirements, constraints, and benchmarking for reference to help build designs that’re included, and are aware they’re not necessary, feel free to disregard them.	5

1	Prioritized Design Criteria	5

1.1	Functional Requirements	5

1.1.1	Table 1.0:  Functional Requirements	5

1.2	Non-Functional Requirements	6

1.2.1	Table 1.1: Non-Functional Requirements	6

1.3	Constraints	7

1.3.1	Table 1.2: Constraints	7

2	Benchmarking	7

2.1	Updated user benchmarking	7

2.1.1	MicroBot	7

2.1.2	Igus	8

2.1.3	X-Series Robotic Arms	8

2.1.4	RobotDigg 6 DOF Coffee Make Robot Arm	8

2.1.5	Lynxmotion	8

2.2	Technical Benchmarking	8

2.2.1	8

3	Target Specifications	9

3.1	Table 1.3: Target Specifications	9

4	Subsystems	11

4.1	Subsystem 1-End-effector	11

4.2	Subsystem 2-Base	11

4.3	Subsystem 3-Arm	11

4.4	Subsystem 4-Controller	11

5	Benefits and Drawbacks of Each Subsystem’s Concepts	11

6	Subsystem Sketches	12

6.1	Subsystem 1-End-effector Sketch	12

6.1.1	Sketch 1	12

6.1.2	Sketch 2	12

6.2	Subsystem 2-Base Sketch	13

6.2.1	Sketch 3	13

6.2.2	Sketch 4	13

6.3	Subsystem 3-Arm Sketch	14

6.3.1	Sketch 5	14

6.3.2	Sketch 6	14

6.4	Subsystem 4-Controller Sketch	15

6.4.1	Sketch 7	15

7	Benefits and Drawbacks of each fully functional solution	15

7.1	Picture 1: Fully Functional Solution 1	15

Picture 2: Fully Functional Solution 2	16

7.2	Picture 3: Fully Functional Solution 3	17

8	Selection Matrix	17

9	Final Sketch	18

9.1	Sketch 12(Final)	18

9.2	Justification	18

10	Wrike Snapshot Link: https://www.wrike.com/frontend/ganttchart/index.html?snapshotId=YOywvLncXxCGCbD9XFwpRTveiykYJJ3o%7CIE2DSNZVHA2DELSTGIYA	18

11	Bibliography	19

# *DISCLAIMER*: We included functional, non-functional requirements, constraints, and benchmarking for reference to help build designs that’re included, and are aware they’re not necessary, feel free to disregard them.

# Prioritized Design Criteria

## Functional Requirements

### Table 1.0:  Functional Requirements

| Number | Need | Design Criteria | Verification Method | Relative importance(1-5, with 5 being most important) |
| --- | --- | --- | --- | --- |
| 1 | Be able to hold a reasonably sized camera, pen, and nozzle. | Clarity of purpose | Test | 5 |
| 2 | The robot head (end effector) can be attached and detach. | Detachable | Test | 3 |
| 3 | Having a 3 DOF arm to enable better movement | Technical need | Test | 4 |
| 4 | Have water blasting end-effectors | Technical need | Test | 4 |
| 5 | Work automatically | Automation | Test | 4 |
| 6 | Using a vacuum and disposable container | Technical need | Test | 2 |

## Non-Functional Requirements

### Table 1.1: Non-Functional Requirements

| Number | Need | Design Criteria | Verification Method | Relative importance (1-5, with 5 being most important) |
| --- | --- | --- | --- | --- |
| 1 | Ability to shut off once it senses the motion of people walking around | Safety | Test | 5 |
| 2 | Ability to withstand high-pressure nozzles to water blast corrosion | Corrosion resistance | Test | 4 |
| 3 | Minor repairs and modifications every three months, and major repairs and modifications every 6 to 12 months. | Product life | Test | 2 |
| 4 | Precise in its movement | Quality | Test | 4 |
| 5 | Identify the nearby space to ensure it completes its programmed tasks. | Robot ability | Test | 3 |
| 6 | System should be built off a common language such as c++, python, GRBL, etc. | Operation code | Test | 2 |
| 7 | Can be use by people with little experience | User friendliness | Test | 3 |
| 8 | Less power consumption | Efficiency | Analysis and Test | 3 |

## Constraints

### Table 1.2: Constraints

Engineering Design: Robotic Arm Prototype – Constraints

| Constraint | Description | Rough Estimates | Verification |
| --- | --- | --- | --- |
| Dimensions (size) | Length, height, and width of the arm are limited to a small and compact environment (ex. Pipe configurations of ship) | Size when deployed: <1m2 (approximately <10.8 ft) Size when Collapse: <1m x 1m x 1m (approximately <35.3 ft3) | Analysis |
| Weight | Must be light weight, easy to carry around in a tight space (ex. Up and down the ladders) | <1 kg (approximately 750 g) | Analysis |
| Time | Scheduled to be completed within three months. The deadline is March 31st | March 31st (approximately < 2 months from now) | Certain |
| Cost | Spend a budget that is less than $100 | <$100 | Estimate, check and review |

# Benchmarking

## Updated user benchmarking

### MicroBot

Compared to the Microbot, including a twisting and tilting axis within our 3 DOF abilities would enhance movement and flexibility. Ensuring stability and precision is something other robot arms look for and would be beneficial to include in ours. Easy use and being open-source increases simplicity when being used by non-technically inclined employees. Microbot has the ability to record and replay complex motion which is an ability we could include in our design. The Microbot uses Blockly, G Code, and Python in the Software, and uses C C++ and JAVA while programming API. These are useful abilities we could include in our design.

### Igus

Igus is a robotic arm meant for material handling and assembly. It can perform tasks like welding, painting, and drilling. There are 4 or 5-axis robotic arms with self-lubricating, maintenance-free plastic joints. The robotic arm has a payload up to 30N, cycle times from 6 seconds, and reach up to 790mm. This is useful to know for our project to get an idea of how much force our arm can stand and how far it could be able to reach.

### X-Series Robotic Arms

This robotic arm is rigid, lightweight, contains T-Slot Aluminum Extrusions, and has a 90-degree elbow. Its reach is 550mm, with 360-degree rotation and a total span of 1100mm. The industrial grade bearing arm has a working payload of 150g. There are 2 different mounting solutions available, wood screws, or thumb screws and pem nuts. This is beneficial information for our project because we can use the ideas of mounting solutions through using similar screws to what the ReactorX 200 Robot Arm has.

### RobotDigg 6 DOF Coffee Make Robot Arm

The coffee making robot arm has 6 DOF and a 24V DC power supply. The payload is 1 kg, max load is 2 kg, and arm length of 650 mm. One thing this robot arm specifies that could help that we haven’t seen yet is the max moving speeds. These speeds are 160 degrees/s for the JS(Rotary), 130 degrees/s for the JL(Upper Arm), and for the lower arm, arm rotary, JB(Arm Swing), and the JT(Arm Swing)’s is 200, 300, 400, and 500, respectively. The material used is aluminum which could be a great option for out design.

### Lynxmotion

This robotic arm has 4 DOF and a payload of 100g. The power supplies would be beneficial to know when completing our target specifications due to it having a lower degree of freedom than most robotic arms which usually have 6. The power supply in is 110V to 240V.

## Technical Benchmarking

Companies such as MicroBot, Universal UR3, and BCN3D follow very similar technical requirements as our naval robotic arm. These requirements consist of the arms being in a small form factor and can help out with numerous small tasks that directly meet our practical needs. BCN3D is very similar to our robotic arm as they also 3D-print their robotic arms which we can use as a benchmark to know what materials we could use to build the prototype to meet our safety and creativity needs. Both the MicroRobot and BCN3D use open-source code which should be very useful in our case because it can help our practical and efficiency needs as anyone can improve our code and functionality of the robotic arm. The Universal UR3 robotic arm is a good reference to how it meets our safety needs. As stated, “The UR-3 can operate directly alongside workers without the need for any barriers due to its enhanced safety features''(Robots Done Right,2022). Our robotic arm being on a ship must be safe to be around crew members so using the Universal UR3 as a technical benchmark should be beneficial to our design and safety needs. Looking at a robotic arm called Meca500, it is claimed that it is the most precise robotic arm on the market with a repeatability of 0.005 mm. This is achievable due to its 6-axis movement (DOF). Trying to implement 6 DOF should be pondered about while creating our prototype as it can easily meet our requirement for essential accuracy for our technical needs.

# Target Specifications

## Table 1.3: Target Specifications

|  | Relation | Value | Units | Verification Method |
| --- | --- | --- | --- | --- |
| End Effector Mass | < | 750 | g | Analysis, final test |
| End Effector Communications Protocol | = | Choice of Serial (UART) for more complex tools or PWM for simpler tools. | N/A | Test |
| Possible types of end effectors (non exhaustive) | = | Grippers, cameras, and pens. | N/A | Test |
| Microcontroller | = | Arduino Uno (ATmega328P) | N/A | Test |
| Arm Degrees of Freedom | = | X-axis translation | N/A | Test |
|  | = | Y-axis translation | N/A | Test |
|  | = | X-axis rotation | N/A | Test |
| Arm Range of Motion | > | 30 | cm | Test |
|  | > | 30 | cm | Test |
|  | = | 180 | degrees | Test |
| User interface | = | Programmable from computer using USB connection. | N/A | Test |
| Cost | < | 100 | $ | Certain |
| Operating temperature | between | -20 and 50 | °C | Test |
| Waterproofing | = | Water resistant, do not submerge. | N/A | Test |
| End Effector Mass | < | 750 | g | Analysis, final test |
| End Effector Communications Protocol | = | Choice of Serial (UART) for more complex tools or PWM for simpler tools. | N/A | Test |
| Possible types of end effectors (non exhaustive) | = | Grippers, cameras, and pens. | N/A | Test |
| Microcontroller | = | Arduino Uno (ATmega328P) | N/A | Test |
| Arm Degrees of Freedom | = | X-axis translation | N/A | Test |
|  | = | Y-axis translation | N/A | Test |
|  | = | X-axis rotation | N/A | Test |
| Arm Range of Motion | > | 30 | cm | Test |
|  | > | 30 | cm | Test |
|  | = | 180 | degrees | Test |
| User interface | = | Programmable from computer using USB connection. | N/A | Test |
| Cost | < | 100 | $ |  |
| Operating temperature | between | -20 and 50 | °C | Test |
| Waterproofing | = | Water resistant, do not submerge. | N/A | Test |

# Subsystems

## Subsystem 1-End-effector

Brooke: The robot head (end effector) can be attached and detached.

Brooke: Have water blasting end-effectors

Brooke: Aluminum end-effector for the prototype to ensure it’s less expensive

Christopher: Identify the nearby space to ensure it completes its programmed tasks through holding a camera

Jacky: be able to hold a pan and nozzle.

Ancelin: Should work at an appropriate speed.

Hassan: High quality materials than can accomplish everyday tasks

## Subsystem 2-Base

Brooke: Ability to withstand high-pressure nozzles to water blast corrosion through a stable base with reliable bolts, like the X-Series Robotic Arm

Jacky: provide strong support for the arm when the arm is making lots of movement; the base is easy to fix.

Ancelin: Strong material for base to make sure it’s stable

Hassan: Ability to clamp itself on a table

Christopher: Wider base to ensure stability

## Subsystem 3-Arm

Ancelin: The arm should be lightweight and easy to transport. When stored the arm should occupy the least amount of volume available.

Ancelin: Assembly should be fast and easy to set up.

Brooke: Arm is precise in its movement

Jacky: Having a 3 DOF arm to enable better movement

Christopher: Should be easy to assemble and repair

Hassan: 3D printer material should withstand forces on the arm

## Subsystem 4-Controller

Brooke: System should be built off a common language such as C++, python, GRBL, etc, just like the MicroBot

Brooke: Code is precise in its movement

Ancelin: The arm should work automatically.

Hassan: Can be used by people with little experience

Jacky: be able to shut off the arm when people are nearby.

Christopher: Arduino based

# Benefits and Drawbacks of Each Subsystem’s Concepts

The pros of our concepts are that we included safety factors and functions that’ll make its performance more efficient and friendly to use. The pros include functional requirements of ensuring it can withstand pressures from the water gun and holding pens, having durable material, and moving precisely through small movements. Having clamps and an easy assembly and maintenance is very useful in our concepts. Although, we have some cons, which includes not having specific plans for each great idea. Once we develop more precise plans to have these concepts in our design, we will then be able to have a functional robotic arm.

# Subsystem Sketches

## Subsystem 1-End-effector Sketch

### Sketch 1

### Sketch 2

## Subsystem 2-Base Sketch

### Sketch 3

### Sketch 4

## Subsystem 3-Arm Sketch

### Sketch 5

### Sketch 6

## Subsystem 4-Controller Sketch

### Sketch 7

# Benefits and Drawbacks of each fully functional solution

## Picture 1: Fully Functional Solution 1

Benefits: This design has a stable base, with a thicker arm that enhances durability and would make it able to withstand higher pressures. The claw can hold a circular pen through clenching down which makes it able to tightly hold onto any pen that has a diameter less than the open claw diameter. The first DOF is a ball which increases movement abilities and would be ideal to make the base connector a ball in the final design.

Drawbacks: This claw is bigger which makes it harder to move around and store, a smaller and less heavy-duty arm that is still durable would be more ideal. The size would use more material and make the arm more expensive to make.

## Picture 2: Fully Functional Solution 2

Benefits: This robotic arm uses less material because it is thinner and also durable due to there being 2 semi-arms to make up for the material in between. This also is beneficial for small storage spaces. This claw can move 180 degrees in the y-axis and can rotate all the way around to its left and right. This will increase the ability to work in difficult areas with lots of bending pipes. The 2 gears on the end-effector controls the claw opening and closing and allows the claw to clench down on several sizes of pens and water guns.

Drawbacks: The circular bottom may make it harder to store since you cannot be effectively stored next to square objects without wasting space. The claw is thinner which may affect its ability to withstand force and weight, which will cause for repair.

## Picture 3: Fully Functional Solution 3

Benefits: This end-effector is thicker which will allow it to withstand stronger forces and weights without breaking. The grooves in the claw will allow the claw to have more grip on the objects it holds. The arm is thinner which decreases materials and prices. The base is wide which will allow the robotic arm to be stable and avoid tipping over. The clamp is beneficial for clamping to tables etc. in order to paint and blast in several areas. The base is square which makes storage and stacking easier.

Drawbacks: The straight inner claw may constrain the claw from clenching down on cameras and pens without damaging them. The claw is thick which increases the price and amount of material. The base connector can only go back and forth, not 360 degrees like a ball can which decreases movement abilities.

# Selection Matrix

# Final Sketch

## Sketch 12(Final)

## Justification

We developed this design through incorporating the best ideas from each fully functional solution, and not using their drawbacks. For example, in the first sketch, we used the idea of having ball joints and a similar claw because its more effective. The circular bases were cut from our arm due to storage constraints within the design. The 2nd sketch had gears in the claw with grips on the tip and not the whole claw. This is beneficial because it can hold a pen due to the claws circular shape, and also grab a camera with the grips on the ends. The 3rd designs claw was too thick and wouldn’t be as flexible and precise, but we liked the square base for more efficient storage (aka, can fit in corners, next to, and in boxes). The ball joints allow for more movement that can be a lot more precise. The arm will be one piece that isn’t too thick like the first sketch, or too complicated like the 2nd sketch. Overall, taking the best parts of the 3 sketches and combining them made for a great final design in our project.

# Wrike Snapshot Link: https://www.wrike.com/frontend/ganttchart/index.html?snapshotId=4gC9n1jwAJ2W8VYUEyNUHDwwR9laY1o4%7CIE2DSNZVHA2DELSTGIYA

# Bibliography

Page 7-arm, 6.-a. m.-i. (n.d.). https://www.youtube.com/watch?v=SisrRUX_Zfk. Retrieved from Mirobot .

Page 7-Automate. (n.d.). The MECA500 is the world's most precise six-axis robot. Retrieved from https://www.automate.org/products/electromate-inc/mecademic-meca500

Page 6-igus. (n.d.). Industrial Robotic Arm and Robotic Components . Retrieved from https://www.igus.com/info/robotic-arm-and-components

Page 7-Lynxmotion. (n.d.). Retrieved from https://www.google.com/shopping/product/7355458352155579770?q=robot%2Barm&prds=epd%3A2030558946649242245%2Ceto%3A2030558946649242245_0&sa=X&ved=0ah

Page 6-MicroBot. (n.d.). Retrieved from https://youtu.be/SisrRUX_Zfk.

Page 7-Mirobot. (n.d.). Mirobot - 6-Axis Mini-Industrial Robot Arm Now Available to the Public. Retrieved from https://www.prnewswire.com/news-releases/mirobot---6

Page 7-RobotDigg. (n.d.). Robot arm six degrees of freedom 6 DOF roboseed. Retrieved from https://www.robotdigg.com/product/1463/Robot-Arm-six-degrees-of-freedom-6-DOF-RoboSEED?gclid=Cj0KCQiAgP6PBhDmARIsAPWMq6m0wwbiutBXhWutXp_OBJ7fPUuynHizRZ00e

Page 7-Robotics, M. (n.d.). The world's smallest, most precise and compact six-axis robot arm. Retrieved from https://www.mecademic.com/en/meca500-robot-arm

Page 7-Robotshop. (n.d.). ReactorX 200 robot arm - robotshop . Retrieved from https://www.robotshop.com/ca/en/reactorx-200-robot-arm.html

Page 7-Technologies, B. (n.d.). A fully open source 3D printed robot arm. Retrieved from https://www.bcn3d.com/bcn3d-moveo-the-future-of-learning-robotic-arm/

Page 7-UR3, U. (n.d.). Robots Done Right. Retrieved from https://robotsdoneright.com/Universal/Universal-UR3.html